/*

4x4x4 LED cube

*/

int column[] = {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15}; // LED column pins

int layer[] = {16, 17, 18, 19}; // LED layer pins

// Setup the column and layer pins as outputs, and make sure they are turned off.

void setup(){

// Setup LED columns.

for(int pin = 0; pin < 16; pin++){

pinMode(column[pin], OUTPUT);   // Set each column pin as an output.

digitalWrite(column[pin], LOW); // Make sure each column pin is turned off.

}

// Setup LED Layers as output

for(int pin = 0; pin < 4; pin++){

pinMode(layer[pin], OUTPUT);   // Set each layer pin as an output.

digitalWrite(layer[pin], LOW); // Make sure each layer pin is turned off.

}

}

// Now start the LED display.

void loop(){

stepLayers(); // This is the display function from class.

incrementLayers(); // This is an alternative display function.

}

// This is the code we wrote in class.

void stepLayers(){

int pause1 = 500;

int pause2 = 250;

while(pause1 > 30){ // A while loop will run as long as the condition in the parentheses is true.

for(int pin = 0; pin < 4; pin++){

if(pin == 3){

digitalWrite(layer[pin], HIGH);

delay(pause2);

digitalWrite(layer[pin - 1], LOW);

delay(pause1);

digitalWrite(layer[pin], LOW);

}

else{

digitalWrite(layer[pin], HIGH);

delay(pause1);

digitalWrite(layer[pin + 1], HIGH);

delay(pause2);

digitalWrite(layer[pin], LOW);

}

}

pause1 -= 30;

pause2 -= 15;

}

}

// This function increments the layers from bottom to top.

void incrementLayers(){

// Turn on the columns. The LEDs will not turn on, because the layers are off.

for(int pin = 0; pin < 16; pin++){

digitalWrite(column[pin], HIGH);

}

// Turn on each layer, one at a time. Repeat the process, going faster each time.

int pause = 1000;

while(pause > 0){

digitalWrite(layer[0], HIGH);

delay(pause);

digitalWrite(layer[0], LOW);

digitalWrite(layer[1], HIGH);

delay(pause);

digitalWrite(layer[1], LOW);

digitalWrite(layer[2], HIGH);

delay(pause);

digitalWrite(layer[2], LOW);

digitalWrite(layer[3], HIGH);

delay(pause);

digitalWrite(layer[3], LOW);

pause -= 20;

}

}